TASK 2

RC Transmitter and Receiver: Manual pilot control (takeoff/landing, emergency override if autonomous logic fails).

Telemetry Module: Two-way link (air + ground radio pair) carrying MAVLink data like GPS position, battery voltage, altitude/speed, mission status to your ground control software (QGroundControl/Mission Planner), and lets you push new waypoints or abort commands up to the drone.

VTX: Streams live video to the ground so a human can visually confirm anything the onboard AI flags as a possible detection, and gives the pilot FPV situational awareness if they take manual control.

For the RC link, I would select a digital, long range protocol like ExpressLRS (900 MHz "LR" mode) with SBUS output. Over open water, with no obstructions ELRS at 900 MHz can realistically hold a link out to 20–30+ km with a decent antenna.

For the Telemetry link, I would choose A Holybro SiK Telemetry Radio V3 or RFD900x (915 MHz). They have native MAVLink support, plug-and-play with Pixhawk's TELEM port, and RFD900x specifically can reach ~40 km over water with good antennas.

For VTX, I would choose a digital system instead of an analog one, since the analog one can cause problems when we need to detect a small target. A digital system like DJI O3 Air Unit gives you HD video out to ~10 km.

If the digital vtx system is a problem for the given weight budget, we can also skip the vtx entirely and send low resolution snapshots using the OAK-D LITE over the telementry link.

Connections:

ELRS receiver  Pixhawk 6C mini

Port: Dedicated RC IN port

Signal Type: SBUS

telemetry radio  Pixhawk 6C mini

Port: TELEM1

Signal Type: MAVlink

Raspberry Pi 5  Pixhawk 6C mini

Port: TELEM2 or USB

Signal Type: MAVlink

OAK-D LITE  Raspberry Pi 5

Port: USB3

Signal Type: Depth + RGB video

FPV camera  VTX

Port: Direct video-in cable

Signal Type: Digital HDMI

Power Module  Pixhawk 6C mini

Port: POWER1 Port

Signal Type: Regulated 5V + voltage/current sense

Battery  Motors

Port: Direct via PDB

Signal Type: High-current power

Where things get power from:

Main LiPo battery → Power Module → Pixhawk POWER1

ESCs/motors draw directly off the battery through the power distribution board (PDB), not through Pixhawk

Raspberry Pi 5 should get its own dedicated 5V/5A UBEC off the main battery

RC receiver is typically powered straight off the Pixhawk RC port pin

Telemetry radio runs off a 5V BEC

VTX usually wants a higher voltage input (7–24V) and is best run off its own regulated tap on the PDB

Protocols and ranges:

RC control: protocol  ExpressLRS (900 MHz LR mode), SBUS to FC, Typical range  20-30+ km

Telementry: protocol  MAVLink over SiK/RFD900 (915 MHz), Typical range  up to 40km

Video: protocol  Digital, Typical range  10km, HD

Companion Computer: protocol  MAVLink over UART/USB, Typical range  NA, wired.